DETAIL ESTIMATE FOR CONSTRUCTION  OF

DAY CARE CENTER FOR AGED  BUILDING OF

COUNCIL  FOR  SOCIO  ECONOMIC  BENEVOLENT  ACTION,  (SEBA)

( Over Plot No. 732/1164/1281, Khata No. 202/57,

Mouza - Jatipur, Baripada in Mayurbhanj District )

At- Walliganj,  Po- Bhanjpur,  Baripada,                                                                          Mayurbhanj, Odisha, India, Pin- 757002.

Date  of  preparation

of  Estimate                 :    20   November  2011

ESTIMATE   FOR  CONSTRUCTION  OF   DAY CARE CENTER

FOR AGED  BUILDING

( Over Plot No. 732/1164/1281, Khata No. 202/57, Mouza - Jatipur, Baripada in Mayurbhanj District )

Center  line  length  :

10"  thick  wall  :  4 x 15'.10" + 2x 36' . 6"  =  136'.4"

5" 	  thick  wall  :   1 x 6'.10"                        =    6'.10"

1.	Earth work in excavation of foundation within

initial lead & lift including dressing & levelling

of bed as per the direction of Engg. in Charge.

10 x 4' - 6" x 4' - 6" x 4' - 6" 					=  911.25 cft

82' - 4" x 1' - 6" x 1' - 6" 					=  185.24 cft

Total	= 1096.49 cft

or say 31.04 cum @ Rs. 5200.00 / 100 cum		               Rs. 1614.08

2.	Filling in foundation & Plinth with sand well

watered and rammed complete.

10 x 4' - 6" x 4' - 6" x 1'-0" = 202.50 cft

82' - 4" x 1' - 6" x 0' - 6"     =   61.75 cft

Foundation Filling :

2/3 x 1096.49 cft = 730.99 cft

Rooms :

6' - 6" x 6' - 0" 	=   39.00 sft

8' - 0" x 6' - 0" 	=   48.00 sft

15' - 0" x 20' - 0" 	= 300.00 sft

15' - 0" x 7' - 0" 	= 105.00 sft

Sub Total	= 492.00 sft   x 1' - 8" =   820.16 cft

Total				   1815.40 cft

or say 51.40 cum @ Rs. 335.72 cum				Rs. 17,256.00

3.	Cement Concrete ( 1:3:6) with 4 cm

size h.g. metal including curing cost

& conveyance etc. complete.

10 x 4' - 6" x 4' - 6" x 0' - 6" 	= 101.25 cft

133' - 2" x 1' - 6" x 0' - 6" 	=   99.88 cft

Rooms  :

492.00 sft x 0' - 4"        	= 163.84 cft

364.97 cft

or say 10.33 cum  @ Rs. 3360.80 / cum			Rs. 34,717.06

cont   …   p2

//  2  //

4.	Brick work using K.B. Bricks having crushing

strength between 75 Kg/cm2 to 99 Kg/cm2 in

C.M. ( 1:6) in F & P including curing, cost,

conveyance etc. complete.

133' - 2" x 0' - 10" x  2' - 0" 		= 	221.86 cft

or say 6.28 cum @ Rs. 1432.90/ cum				Rs. 8,999.00

5.	Brick work with above specifications in super

structures including curing cost & conveyance

etc. complete.

126' - 4" x 0' - 10" x 9' - 4" =  982.16 cft.

6' - 10" x  0' - 5"  x 9' - 4" =    26.57 cft.

Head Room :

2 ( 15' - 0" + 7' - 0" ) x 0' - 10" x 7' - 6 " =  274.89 cft

Parapet Walls :

(2 x 26'-0" +25'-0") x 0' - 10" x 3' - 0" = 167.43 cft

1451.05  cft

Deduction :

Doors

1 x 4' - 0"  = 4' - 0"

1 x 3' - 6"  =3' - 6"

2 x 3' - 0"  =6' - 0"

13' - 6" x 0' - 10" x 6' - 6" = 73.09  cft

Windows :

2 x 5' - 0"  =10' - 0"

2 x 4' - 0"  =  8' - 0"

2 x 3' - 0"  =  6' - 0"

24'-0" x 0' - 10" x  4' - 0"  = 79.97   cft

Ventilator - 2 x 1'-6"x 2' -0" x 0' -10"  =   5.00   cft

Grill  gate - 1 x 5' - 0"x 0'-10" x 7'-0"  =  29.15  cft 						                                                                187.21  cft

Net Brick Work  :   1451.05 cft - 187.21 cft  = 1263.84 cft

Or  say 35.78 cum  @  Rs. 2991.70 / cum	   	          Rs. 107043.00

6.	12mm thick C.P. ( 1:4) with neat cement

punning & two coats of bitumen painting

over top of walls including cost, curing

& conveyance etc. complete.

126' - 4" x 0' - 10" = 105.23 sft.

6' -10" x 0' - 5"   =     2.84 sft

cont   …   p3

//  3  //

44'- 0" x 0' -10"   =   36.65  sft

Total              144.72   sft

or say 13.45 sqm @ Rs. 59.90 / Sqm				Rs.    805.65

7.	Cement concrete ( 1:2:4) using 12mm thick

size h.g. chips for R.C.C. roofing including

curing, cost and conveyance etc. complete.

10 x 4' - 6" x 4' - 6" x 0' - 9" = 151.87 cft

10 x 1-3"  x { ( 4'' - 6" ) 2 + (1'-3")2 + 4 (4' - 6" + 1' -3" ) 2 }= 114.32  cft

6                        		 	    2

Total                                  266.19 cft

or  say   7.54 cum  @  Rs. 4380.70 / cum			Rs. 33030.50

8.	Cement concrete (1:2:4) using 12mm size

h.g. chips for R.C.C. columns, because &

lintels including curing, cost, conveyance

centering & shuttering etc. complete.

Columns      - 10 x 0' - 10" x 0' - 10" x 11' - 4"    	= 78.64 cft

Tie beams    - 143' - 2" x 0' - 10"x 1' - 0" 		= 119.26 cft

Roof beams - 168' - 2" x 0' - 10" x 0' - 8" 		= 93.43 cft

Lintels         - 143' - 2" x 0' - 10" x 0' - 6" 		= 59.63 cft

Total 	=350.96 cft

or say  9.94 cum @ Rs. 6801.80 / cum			Rs. 67609.90

9.	Cement concrete ( 1:2:4) using 12mm size h.g. chips

for roof slab, chajja & loft etc. including curing, cost,

conveyance, centering and shuttering etc. complete.

Roof Slab -  29' - 0"  x 21' - 2" x  0' - 4"    = 204.41  cft

Chajja       -  36'- 0"   x  2' - 0"  x  0' - 2"    =  12.02   cft

Total	216.43  cft

or say 6.13 cum  @ Rs. 5586.60/cum				Rs. 34245.80

10.	Cement concrete ( 1:2:4) with 12mm size h.g.b

chips for R.C.C. stairs & landing etc. including

curing , cost, conveyance, centering & shuttering

etc. complete.

20' - 0" x 3' - 6" x 0' - 5"		 =	29.16   cft

20x 1/2 x 0' - 10" x 0' - 6 " x 3' - 6 "    =	14.58   cft

cont   …   p4

//  4  //

2 x  5' - 2" x 8' - 8" x 0' - 5"		=	37.31  cft

Total    	81.05  cft

Or  say  2.29 cum   @  Rs. 5586.60 / cum		   	Rs. 12793.30

11.	Cutting bending, binding, straightening laying the

grills & placing in position including cost of binding

wire 18 to 20 gauge & cost of rods complete.

Footing   - 7.54 cum x 0.60 qntl / cum 	= 4.52 qntl.

Columns - 2.23 cum x 1.20 qntl/cum  	= 2.68 qntl

Beams     - 7.71 cum x 1.40 qntl/ cum 	= 10.79 qntl.

Roofslab - 613.84 sft x 0.0085 qntl / sft 	= 5.22 qntl

Chajja     - 72.00 sft x 0.0080 qntl/sft 	= 0.58 qntl

Stairs      - 2.29 cum x 1.40 qntl/ cum 	= 3.21 qntl.

27.00 qntl.

27.00 qntl   @  Rs. 4600.00 / qntl				Rs. 1,24,200.00

12.	12mm thick cement plaster (1:6) over brick walls racking

out joints including cost & conveyance etc complete.

Outside -

2 (16' - 8 + 37' - 4" ) x 13' - 0"   = 1404.00  sft

Headroom –

2 (16' - 8" + 8' - 8" ) x 7' - 6"     =   350.01  sft

Paperpet wall :-

70' - 4"   x   6' - 10"                   =  480.58  sft.

Inside -

2 ( 6' - 6"  +  6' - 0" ) 	= 25' - 0"

2 ( 8' - 0"  +  6' - 0" ) 	= 28' - 0"

2 (15'- 0"  + 20'- 0" ) 	= 70' - 0"

123' - 0" x 10' - 6"  = 1291.50  sft

Head  Room

2 ( 15' - 0" + 7' - 0" )  x 18' - 0"                   =    792.00  sft

Total              2083.50   sft

Deduction :

Doors

13' - 6" x   6' - 6"   = 87.75 sft

Windows :

24' - 0" x  4' - 0"   = 96.00 sft

Ventilators

3' - 0"  x  2' - 0"   =  6.00  sft

Grill  gate

5' - 0"  x  7' - 0"   = 35.00  sft

Total           224.75 sft

cont   …   p5

//  5  //

Net Plastering 	= 2234.59 + 2083.50  - 224.75  sft  = 4093.34  sft

Or  say   380.42  sqm   @  Rs.  59.90/ sqm			         Rs. 22,787.15

13.	12mm thick ceiling plaster with C.M (1:4) on roof & chajja

including curing cost and conveyance etc. complete.

Qnty. vide Item - 9 & 10

Roof Slab -	 613.84 sft

Chajja       - 	   72.00 sft

685.84  sft

or  say  63.74  sqm  @  Rs. 64.30 sqm				Rs. 4,104.85

14.	25mm thick AS flooring with  CC 1:2:4  using 12mm

size  HG  chips  including cost, curing and conveyance etc.

Qnty vide item  no 2’ - 492.00   sft

or say  45.72  sqm  @  Rs. 153.80 / sqm				Rs. 7031.74

15.	12mm thick cement plaster with C.M (1:4) with neat cement punning

for skirting  including curing cost and conveyance etc. complete.

123' - 0" + 44' - 0" -   18' - 6" ) x   0' - 9"   = 111.37  sft

or  say  10.35  sqm  @  Rs. 68.70   sqm				Rs.    711.04

16.      Dressed  seasoned  sal wood work  framed  &  fixed in frames

of  doors  &  windows .

Doors :

1 x  17' - 0"       =   17' - 0"

1 x  16' - 6"       =   16' - 6"

2 x  16' - 0"       =   32' - 0"

Windows  :

2 x  18' - 0"       =   36' - 0"

2 x  16' - 0"       =   32' - 0"

2 x  14' - 0"       =   28' - 0"

161' - 6"  x  0' - 3" x 0' - 4"  =  13.444   cft

or  say   0.38   cum  @  Rs. 36,370.00 / cum			Rs.    13820.00

cont   …   p6

//  6  //

17.	Fitting & fixing of steel shutters & frame

should be made of M.S. angle including

cost of all materials etc. complete.

Doors - 	13' - 6" x 6' - 6"		= 87.75 sft

Windows	24' - 0" x 4' - 0" 		= 96.00 sft

Total	  183.75 sft

or say   850.00 Kg @ Rs 46.00 /Kg				      Rs. 39,100.00

18.	Cost of ventilators & window grills fitting & fixing

etc. all complete.

Windows     -  24' - 0" x 4' - 0"    = 96.00 sft

Ventilations -    5' - 0" x 7' - 0"    = 35.00 sft

131.00 sft

or say  262.00 Kg @ Rs. 45.00 /Kg			                  Rs. 11,790.00

19.	Painting two coats with approved paint on new wood

work including cost of paints etc. complete.

Qnty vide item no 17’ & 18’

Doors        - 	 87.75 sft x 2.25  = 197.44  sft

Windows  -	 96.00 sft x 2.75  = 264.00  sft

Grill gate  -	 35.00 sft x 0.50  =   17.50  sft

478.94

or say   44.51 sqm   @  Rs. 61.10/sqm			       Rs.  2719.60

20.	Cement washing two coats on walls complete.

Qnty. vide Item no  12’     =  380.42  sqm

Qnty. vide Item no  13’     =    63.74  sqm

Total     444.16  sqm

444.16 sqm   @   Rs.  18.40  /  sqm			       Rs.  8,172.50

21.     Supplying ,fitting & fixing of WI  clamps for doors & windows

with C.C  (1:2:4) including curing , cost & conveyance etc complete .

Doors       :     4  x  6  nos    =   24   nos

Windows  :     6  x  4  nos    =   24   nos

48   nos  @  Rs  25.00  each            Rs     1200.00

cont   …   p7

//  7  //

Add 10% for Sanitary & Water supply fittings

including construction of Septic tank etc. &

overhead tank				L.S.			Rs     40000.00

Add 5% for electrical installations	L.S.			Rs.    20000.00

Add 2% for contingency  & supervision charges L.S.	Rs.     12000.00

Total	Rs. 6,60,198.00

Or say   Rs 6,60,000.00

(  Rupees  six  lakhs sixty   thousand   only  )

Cost  Break up  :

1.  Up to plinth level               :  Rs 1,80,000.00

2.  Up to roof  level                 :  Rs 3,20,000.00

3.  Cost of remaining work     :  Rs 1,60,000.00

## TOTAL COST        :  Rs  6,60,000.00

Estimate  prepared  on Request   :

Baripada

20th   November  2011.

